Name: Darsh Mehta

TE COMPS

Batch B

UID: 2018130023

# CEL 51, DCCN, Monsoon 2020

Lab 4: Prototyping a Network

Objective:

Prototype a network using Packet Tracer

Background

A client has requested that you set up a simple network with two PCs connected to a switch. Verify that the hardware, along with the given configurations, meet the requirements of the client.

Switches facilitate the sharing of resources by connecting together all the devices, including computers, printers, and servers, in a small business network. It connects devices on a computer network by using packet switching to receive and forward data to the destination device. A network switch is a multiport network bridge that uses MAC addresses to forward data at the data link layer of the OSI model.

Router connects multiple switches, and their respective networks, to form an even larger network. It works as a dispatcher, directing traffic and choosing the most efficient route for information, in the form of data packets, to travel across a network.

Step 1: Set up the network topology

- Add two PCs and a Cisco 2950T switch

- Using  straight-through  cables,  connect	PC0	to  interface

Fa0/1 on Switch0 and PC1 to interface Fa0/2 on Switch0.

- Configure PC0 using the Config tab in the PC0 configuration window:

a. IP address: 192.168.10.10

b. Subnet Mask 255.255.255.0

- Configure PC1 using the Config tab in the PC1 configuration window

a. IP address: 192.168.10.11

b. Subnet Mask 255.255.255.0

Step 2: Test connectivity from PC0 to PC1

- Use the ping command to test connectivity.

  - Click PC0.

  - Choose the Desktop tab.

  - Choose Command Prompt.

  - Type: ping 192.168.10.11 and press enter.

- A successful ping indicates the network was configured correctly and the prototype validates the hardware and software configurations. A successful ping should resemble the below output:

- Close the configuration window.

192.168.10.10

192.168.10.11

- Click	the	Check	Results	button	at	the	bottom	of	the instruction window to check your work.

# CEL51, DCCN, Monsoon 2020

Lab 4.1: Basic configuration - hostname, motd banner, passwd etc

Objective:

This lab will test your ability to configure basic settings such as hostname, motd banner, encrypted passwords, and terminal options on a Packet Tracer 6.2 simulated Cisco Catalyst switch.

- Use the local laptop connect to the switch console.

Rename Laptop0 –> Local Laptop

Connect console connection to RS232 port of Local Laptop and Console port of Switch

Open terminal of local laptop

Enable command - To enter in privilege exec mode

- Configure Switch hostname as LOCAL-SWITCH Enter configuration mode

Use the configure privileged EXEC command to enter global configuration mode.

Set hostname as LOCAL-SWITCH using hostname LOCAL-SWITCH command

Run show running-config command to check the hostname.

- Configure the message of the day as "Unauthorized access is forbidden"

Use command banner motd #

Type the message and add # at the end.

You can check the message of the day when you open the terminal for accessing switch again.

- Configure the password for privileged mode access as "cisco". The password must be md5 encrypted

Use command enable secret cisco

In running-config it displays as enable secret.

When we try to enable switch again, it will ask for password.

- Configure password encryption on the switch using the global configuration command

In running-config, service password-encryption is displayed.

- Configure CONSOLE access with the following settings:

- Login enabled

- Password: whatever you like

- History size: 15 commands

- Timeout: 6'45''

- Synchronous logging

The con 0 configuration is empty in the beginning

Line con 0 now shows the console configuration.

- Configure TELNET access with the following settings:

- Login enabled

- Password: whatever you like

- History size: 15 commands

- Timeout: 8’20’’

- Synchronous logging

- Configure the IP address of the switch as 192.168.1.2/24 and its default gateway IP (192.168.1.1).

Ip address and default gateway are now displayed in interface vlan The default gateway address is the ip address of the router.

- Test telnet connectivity from the Remote Laptop using the telnet client.

Configuration of Remote laptop

Configuration of Router

Configuration of Switch

Configuration of Remote Laptop

Pinging Router from Remote Laptop

Pinging another Ethernet port of Router from Remote Laptop

Pinging Switch from Remote Laptop

Telnet Switch from Remote Laptop

After entering password for telnet

Enabling switch from Remote Laptop

Conclusion:

  - In this experiment, I learned about setting up network with Router and Switch.

  - I learned to configure Switch using console. I understood how to configure terminal.

  - I configured telnet for switch and checked its connectivity from remote laptop.